LAB GUIDE & INSTRUCTIONS

Configuring Access Control & Identity Services

Cisco Packet Tracer Lab  |  Module: Technical Development  |  Focus: AAA, Email & FTP Security

Activity Overview & Objectives

Authentication and authorization are distinct, foundational security processes in Identity and Access Management (IAM). Authentication confirms identity through credentials, whereas authorization defines user access privileges to specific resources. In this activity, you will configure centralized AAA authentication for wireless access, secure email client/server infrastructure, and granular role-based permissions on an FTP server.

Part 1: Configure & Use AAA Authentication Credentials: Set up RADIUS accounts and link client laptops to secure enterprise wireless.

Part 2: Configure & Use Email Services: Activate SMTP/POP3 mail services, configure client profiles, and verify bidirectional messaging.

Part 3: Configure & Use FTP Services: Provision FTP user access control lists (ACLs/privileges) and validate file upload, download, and delete restrictions.

LAB ENVIRONMENT TIP: This activity opens in Physical Mode by default, but you may complete it in Logical Mode if preferred. Most items are automatically graded—click 'Check Results' at any point to verify configuration status or trigger network convergence.

Part 1: Configure & Use AAA Authentication Credentials

Step 1: Configure User Accounts on the AAA Server

Navigate to Wiring Closet: From Headquarters (HQ), click on the Wiring Closet (tall, black server chassis in the bottom-left corner).

Access AAA Service: On the right-side rack, click AAA-RADIUS server > Services tab > AAA (under SERVICES).

Enable Service: Set the AAA service toggle to On.

Add User Accounts: Under User Setup, enter the username and password pairs below and click Add (+):

| Username | Password | Target System |
| --- | --- | --- |
| user1 | PASSuser1! | HQ-Laptop-1 Authentication |
| user2 | PASSuser2! | HQ-Laptop-2 Authentication |

Step 2: Configure Wireless Authentication on HQ-Laptop-1

Select Device: Navigate back to HQ. Click HQ-Laptop-1 (located two rooms right of the Wiring Closet).

Access Interface: Click Config tab > under INTERFACE, click Wireless0.

Configure SSID: In the SSID field, type: HQ-INT

Select Authentication Mode: Under Authentication, select WPA2.

Input Credentials: In the User ID field enter user1, and in Password enter PASSuser1!

Verify DHCP Lease: Under IP Configuration, select DHCP. Wait several seconds for the DHCP offer. Confirm the laptop obtains an IP address in the 192.168.50.0/24 subnet.

TROUBLESHOOTING TIP: If DHCP does not assign an address immediately, toggle between 'Static' and 'DHCP' or click 'Check Results' to force Packet Tracer network convergence. You can also configure HQ-Laptop-2 first, then return to verify HQ-Laptop-1.

Step 3: Configure Wireless Authentication on HQ-Laptop-2

Select Device: Click HQ-Laptop-2 (located in the top-right corner of HQ).

Configure Wireless Settings: Navigate to Config > Wireless0. Set SSID to HQ-INT, select WPA2, and authenticate using User ID: user2 / Password: PASSuser2!

Verify IP Assignment: Verify the device receives an IP configuration via DHCP within the 192.168.50.0/24 subnet.

Part 2: Configure & Use Email Services

Step 1: Activate Email Services & Provision User Accounts

Access Mail Server: Navigate to the Wiring Closet. In the right-side rack, click Mail server > Services tab > EMAIL.

Enable Protocols: Turn On both SMTP Service and POP3 Service.

Configure Domain: In the Domain Name field, enter: mail.cyberhq.com, then click Set.

Create User Mailboxes: Under User Setup, enter each username and password below, clicking the plus sign (+) to create each account:

| Account / Username | Password | Associated Client / Role |
| --- | --- | --- |
| HQuser1 | Cisco123! | PC 1-1 (Suk-Yi) |
| HQuser2 | Cisco123~ | Net-Admin PC (Cisco) |
| BRuser1 | Cisco123- | PC 2-3 (Ajulo) |
| BRuser2 | Cisco123+ | HQ-Laptop-1 (Malia) |

Step 2: Configure Client Email Applications

Open each specified device, navigate to Desktop tab > Email (Configure Mail), enter the corresponding profile parameters, and click Save. For all workstations, both the Incoming Mail Server and Outgoing Mail Server (SMTP) must be set to: mail.cyberhq.com

| Device | Display Name | Email Address | User Name | Password |
| --- | --- | --- | --- | --- |
| PC 1-1 (HQ Corner) | Suk-Yi | HQuser1@mail.cyberhq.com | HQuser1 | Cisco123! |
| PC 2-3 (Below Conf) | Ajulo | BRuser1@mail.cyberhq.com | BRuser1 | Cisco123- |
| HQ-Laptop-1 | Malia | BRuser2@mail.cyberhq.com | BRuser2 | Cisco123+ |
| Net-Admin (Closet) | Cisco | HQuser2@mail.cyberhq.com | HQuser2 | Cisco123~ |

Step 3: Test & Verify Email Connectivity

Compose Message: On PC 1-1, open Email and click Compose.

Send Test Email: Address to: BRuser1@mail.cyberhq.com. Add any subject and message body. Click Send. Verify the message bar confirms 'Send Success' (allow a few seconds for DNS/ARP resolution).

Receive & Validate: Navigate to PC 2-3 (Ajulo). Open Email, click Receive, and confirm the test email from Suk-Yi appears in the inbox.

Part 3: Configure & Use FTP Services

Step 1 & 2: Activate Service & Configure Granular Permissions

Activate FTP: Navigate to Wiring Closet > FTP server > Services tab > FTP. Toggle service to On.

Provision Role Permissions: Under User Setup, enter each user, set password, check the required permission flags, and click Add:

| Username | Password | Permissions (R/W/D/N/L) | Authorization Description |
| --- | --- | --- | --- |
| sukyi | cisco123 | [X] Read [X] Write [X] Delete [X] Rename [X] List | Full privileges (RWDNL) |
| ajulo | cisco321 | [X] Read [X] Write [X] Delete [X] Rename [X] List | Full privileges (RWDNL) |
| malia | cisco123 | [X] Read [X] Write [ ] Delete [X] Rename [X] List | Restricted: NO Delete privilege (RWNL) |

Step 3: File Transfer Operations (Net-Admin PC)

Open Terminal: Click Net-Admin PC > Desktop > Command Prompt.

Connect via FTP: Enter command: ftp 192.168.75.2
Authenticate with Username: sukyi | Password: cisco123

List & Download: Enter dir to view directory listing.
Enter get aMessage.txt to download the file to the PC.
Enter quit to terminate the session.

View File Content: Close Command Prompt. Open Text Editor > File > Open > select aMessage.txt.

File Verification Question: What is the message in the file?
→ Answer: 'Greetings. You have successfully accessed the FTP server.'

Create New File: In Text Editor, click File > New. Type a custom message. Save file as: aMessage_new.txt. Close Text Editor.

Upload New File: Return to Command Prompt. Re-connect: ftp 192.168.75.2 (login as sukyi).
Upload using: put aMessage_new.txt
Enter quit when complete.

Step 4: Verify Privilege Enforcement & Access Restrictions

Login as Restricted User: Navigate to HQ-Laptop-1 > Desktop > Command Prompt.
Connect to FTP: ftp 192.168.75.2
Login using Username: malia | Password: cisco123

Test Delete Restriction (Authorization Test 1): Attempt to delete the file: delete aMessage_new.txt
• Observation / Result: Deletion fails with permission denied / access denied error.
• Explanation: malia was not granted the 'Delete' privilege in User Setup.

Test Rename Permission (Authorization Test 2): Attempt to rename the file:
ftp> rename aMessage_new.txt aMessage_rename.txt
• Observation / Result: Operation succeeds.
• Explanation: malia possesses the 'Rename' (N) privilege on the server.

Exit Session: Type quit and close the HQ-Laptop-1 window.

COMPLETION CHECK: Verification complete! Open Check Results in Cisco Packet Tracer to confirm all assessment items reflect 65/65 points.